“THERMOCHEMICAL ANALYSIS OF HYDROGEN PEROXIDE WITH APPLICATIONS TO ROCKET DESIGN”

A PROJECT REPORT

Submitted

in partial fulfillment of the requirements for

the award of degree of

BACHELOR OF ENGINEERING

in

AEROSPACE ENGINEERING

By

| Name | USN |
| --- | --- |
| Bharath R | 1BM22AS011 |
| Chaithra G | 1BM22AS012 |
| Farida Bano | 1BM22AS017 |
| Girish M | 1BM23AS400 |

Under the Guidance of

Dr. Gnanendra. P.M

Assistant professor

Department of Aerospace Engineering

B.M.S. College of Engineering, Bengaluru-560019

Department of Aerospace Engineering

B.M.S. COLLEGE OF ENGINEERING

Autonomous Institution Affiliated to Visvesvaraya Technological University, Belagavi

Post Box No. 1908, Bull Temple Road, Bengaluru-560 019, Karnataka, India.

2024-25

CERTIFICATE

This is to certify that the project work entitled “THERMOCHEMICAL ANALYSIS OF HYDROGEN PEROXIDE WITH APPLICATIONS TO ROCKET DESIGN” is carried out by below mentioned students of B.M.S. College of Engineering in partial fulfillment for the award of Bachelor of Technology in Aerospace Engineering of the Visvesvaraya Technological University, Belagavi during academic the year 2024-25. It is certified that all the corrections / suggestions indicated for the internal assessment have been incorporated in the report. The dissertation report has been approved as it satisfies the academic requirements in respect of project work prescribed by the said degree.

| (Dr. Gnanendra. P.M) | (Dr. K.Veerabhadrappa) | (Dr. Bheemsha Arya) |
| --- | --- | --- |
| Signature of Guide | Signature of HOD | Signature of Principal |

| External VIVA VOCE |  |  |
| --- | --- | --- |
|  | Name of the Examiners | Signature with Date |
| 1. |  |  |
| 2. |  |  |

DECLARATION

I hereby declare that the project work entitled by “THERMOCHEMICAL ANALYSIS OF HYDROGEN PEROXIDE WITH APPLICATIONS TO ROCKET DESIGN” has been independently carried out by us under the guidance of Dr. Gnanendra.P.M, Assistant professor, Department of Aerospace Engineering, B.M.S. College of Engineering, Bengaluru-560019 in partial fulfillment of the requirements of the degree in Bachelor of Technology in Visvesvaraya Technological University, Belagavi. I further declare that, no part of this work has been submitted for any Degree or Diploma of any institution previously.

| Name | USN | Signature |
| --- | --- | --- |
| Bharath R | 1BM22AS011 |  |
| Chaithra G | 1BM22AS012 |  |
| Farida Bano | 1BM22AS017 |  |
| Girish M | 1BM23AS400 |  |

Place: Bengaluru

Date:

ACKNOWLEDGEMENTS

TABLE OF CONTENTS

TABLE OF CONTENTS…………………………………………………………....S

LIST OF TABLES…………………………………………………………………...S

LIST OF FIGURES………………………………………………………………….S

LIST OF SYMBOLS AND ABBREVIATIONS……………………………………S

CHAPTER 1. INTRODUCTION……………………………………………………1

  - OVERVIEW AND BACKGROUND…………………………………..

    - Historical significance of hydrogen peroxide……………...

    - The European Green Deal………………………………….

    - Chemical Equilibrium with Applications………………….

    - Current developments……………………………………..

LIST OF TABLES

| TABLE NO | NAME OF TABLES | PAGE NO |
| --- | --- | --- |
| TABLE 1 | R-phrase categories for hydrogen peroxide, hydrazine and (MMH) |  |
| TABLE 2 | Various parameters values at Chamber, throat and exit |  |
| TABLE 3 | Performance Parameters |  |
| TABLE 4 | Mass Fraction |  |
| TABLE 5 | Tabulation for O/F, Isp and Exit temperature |  |

LIST OF SYMBOLS AND ABBREVIATIONS

CHAPTER 1. INTRODUCTION

1.1 Overview and Background

1.1.1 Historical significance of hydrogen peroxide

Hydrogen peroxide has long been known to be a useful energy source. During World War II Germans experimented and later developed rockets, underwater craft and aircrafts, powered with hydrogen peroxide. The use of hydrogen peroxide as a rocket propellant can be traced back to Helmuth Walter and the Heinkel, He 52 aircraft, which was first flown in 1937. Now it is applied to monopropellant thrusters, liquid rocket and torpedoes engines, hybrid rocket systems, and other propulsion applications as a non-toxic propellant.

This work implements the optimal initial hydrogen peroxide decomposition parameters to produce the superheated steam and oxygen according to the principal potential hazards of concentrated hydrogen peroxide. Most of the published data represent the decomposition analysis of a propellant—grade hydrogen peroxide, while the less concentrated hydrogen peroxide water solutions are less studied.

Hydrogen peroxide (H₂O₂) has garnered significant interest in recent years as a versatile and environmentally friendly oxidizer for propulsion and energy systems. Its decomposition into water and oxygen releases substantial thermal energy, making it a viable candidate for use in various applications, including rocket propulsion, power generation, and chemical processing. The focus on green propellants like hydrogen peroxide is driven by the need to minimize environmental impact and reduce the toxic byproducts typically associated with traditional propellants.

A thermodynamic analysis of hydrogen peroxide involves understanding its behaviour under varying temperature and pressure conditions, its decomposition process, and the energy outputs associated with these reactions. Such an analysis provides critical insights into the efficiency, performance, and feasibility of utilizing hydrogen peroxide as a propellant or energy source.

Hydrogen peroxide's decomposition reaction is exothermic and can be catalysed to enhance energy release. The reaction follows the simplified chemical equation:

2 H2 O2 → 2 H2O + O 2 + Energy

This process produces high-temperature steam and oxygen, which can be harnessed in propulsion systems for generating thrust. The energy released per unit mass, also known as the specific energy, is a crucial parameter that determines its efficacy compared to other oxidizers.

1.1.2 The European Green Deal.

This approved in 2020, is a set of policy initiatives by the European Commission[1].The overarching aim of the European Green Deal is for the European Union to become the world's first “climate-neutral bloc” by 2050. It has goals extending to many different sectors, including construction, biodiversity, energy, transport and food.[2]

Fig 1. The European green deal structure

The plan includes potential carbon tariffs for countries that don't curtail their greenhouse gas pollution at the same rate.[17] The mechanism to achieve this is called the Carbon Border Adjustment Mechanism (CBAM).[18] It also includes:

- circular economy action plan,[19] The European Commission has released a number of publications on circular economy, including one that requires Member States to carry out activities related to changing their economies into circular economies. The CE has indeed become a key component of the European Green Deal and the Coronavirus Recovery Plan of the Von der Leyen Commission (2019–present), and it was a key component of the Juncker Commission's ambition to create a sustainable, low-carbon, resource-efficient, and competitive economy.[20]

- A review and possible revision (where needed) of the all relevant climate-related policy instruments, including the Emissions Trading System,

- A Farm to Fork strategy along with a focus shift from compliance to performance (which will reward farmers for managing and storing carbon in the soil, improved nutrient management, reducing emissions, ...),

- A revision of the Energy Taxation Directive which is looking closely at fossil fuel subsidies and tax exemptions (aviation, shipping),

- A sustainable and smart mobility strategy and

- An EU Forest strategy. The latter will have as its key objectives effective afforestation, and forest preservation and restoration in Europe.

1.1.3 Chemical Equilibrium with Applications.

To thoroughly evaluate the performance of hydrogen peroxide as a propellant, a detailed thermodynamic analysis is essential. This involves understanding the reaction products, energy release, and the influence of operating conditions such as temperature, pressure, and concentration. A robust and accurate tool for performing such analyses is NASA's Chemical Equilibrium with Applications (CEA) software.

NASA CEA is widely used in aerospace and propulsion research for simulating chemical reactions and determining thermodynamic properties under equilibrium conditions. By leveraging the capabilities of CEA, this study examines the decomposition of hydrogen peroxide, computes thermodynamic parameters such as enthalpy, entropy, and specific impulse, and evaluates the system performance across a range of operating conditions.

This process generates high-temperature steam and oxygen, making it a compelling option for both monopropellant and bipropellant propulsion systems. Using NASA CEA, the reaction is analysed to determine the equilibrium composition of products, adiabatic flame temperature, and specific impulse for varying hydrogen peroxide concentrations and chamber pressures.

The integration of NASA CEA into this thermodynamic analysis ensures precise modelling of the chemical and physical processes, enabling optimization of hydrogen peroxide-based propulsion systems.

1.1.4 Current developments

Hydrazine has proven to be an excellent monopropellant in terms of its performance with a theoretical specific impulse of 230 s [4]; However, hydrazine is a highly toxic and dangerously unstable substance. Importantly, its use as a propellant has been compromised by stringent laws to protect personnel who have to work with substances which are highly toxic and carcinogenic. Recently, much more benign, low toxicity (“green”) storable liquid propellants have therefore attracted considerable attention as possible replacements for hydrazine. This is due to the significant increase in the costs of production, storage and handling of toxic propellants, and has set the stage for the search for green replacement propellants. Hydrogen peroxide is one of the most attractive replacements, not only since it is non-toxic and non-carcinogenic, but also due to its many advantageous properties, such as its high density and relatively low cost. From a financial standpoint, Hydrogen peroxide also promises significant cost savings due to the drastic simplifications in health and safety protection procedures during the production, storage and handling of the propellant. These advantages are of special relevance in low or medium thrust rocket engines for small missions, which offer strong potential as profitable opportunities for focused SMEs [5]. In 2013, Nammo, an international aerospace and defense company headquartered in Norway, was selected for the development and delivery of the secondary propulsion system onboard the Ariane 5 ME second stage. The mono-propellant propulsion subsystem offered by Nammo is based on hydrogen peroxide as the propellant and was selected as a green propulsion alternative to replace hydrazine. After Ariane 5 ME was cancelled in favor of Ariane 6, the development contract of the system was annulled in preparation of renewed requirements to be released by the Prime Contractor during 2015 for Ariane 6. Earlier in 2015, Nammo has done extensive testing of the environmentally friendly propulsion system under in-vacuum conditions at DLR Lampoldshausen’s high-altitude simulation facilities in Germany. The system functioned flawlessly for both short pulses, long duration burning at nominal thrust level of 200 N as well as throttling between 50 N and 230 N. Heat soak and chill down under vacuum conditions were the main focus during these vacuum test firings. Throttling is achieved through a newly developed Flow Control Valve from Moog Ireland Ltd. The Nammo developed secondary propulsion system has demonstrated excellent properties in throttling, pulsing, multiple restarts without igniter and excellent cold start capabilities. Long duration burns result in relatively moderate structural temperatures, which ease integration with the spacecraft.

CHAPTER 2. LITERATURE SURVEY

2.1 Review on research on hydrogen peroxide.

2.1.1 Hydrogen peroxide - A promising oxidizer for rocket propulsion and its application in solid rocket propellants.

This paper discusses the potential use of hydrogen peroxide as oxidizer for solid rocket propulsion. While hydrogen peroxide is a liquid in normal conditions, it may be used in solid rocket motor grains. Use of hydrogen peroxide of HTP class (High Test Peroxide) is proposed. It has been known for many decades but its utilization has been historically limited to liquid propellants. Until recently is application even in liquid state in rocket propulsion for space and defense has been limited due to storability and safety issues. With the availability of new grades of HTP with higher purity and concentrations, enhanced performance, safety and storability are possible. This results in HTP being considered for a wide range of rocket propulsion systems: as oxidizer in bipropellant and hybrid propulsion systems, as well a monopropellant. To ensure proper analysis of the potential of using HTP in next-generation solid rocket propellants, this paper reviews existing rocket propulsion applications of HTP.

2.1.2 Arc-Ignition of a 70% Hydrogen Peroxide/ABS Hybrid Rocket System

Utah State University (USU) teamed with Parabilis Space Technologies, Inc. (Parabilis), on a NASA STTR Solicitation T1.01, Affordable Nano/Micro Launch Propulsion Stages. During this collaboration USU worked on the development and testing of a novel laboratory scale thruster that employs medium grade hydrogen peroxide (70%) and additively-manufactured ABS as propellants. The thruster was adapted from previous hybrid rocket systems designed and tested at USU. In the adapted hybrid thruster design a 70% peroxide solution replaces GOX as the oxidizer; and this solution is catalytically decomposed upstream of the inlet to produce approximately 1/3 gram of molecular oxygen each gram of solution. The liberated GOX is used in conjunction with the existing arc-ignition system to light the motor. Multiple on-demand relights are available with this system. This “green-propellant” system has significant advancements over alternative technologies in the areas of cost, safety, and mission capability.

2.1.3 ISRO working on green fuels like hydrogen peroxide for rockets

ISRO Chairman said that the Indian Space Research Organisation (ISRO) is looking at green propulsion through hydrogen peroxide in its rocket that would take Indians into the space under its ‘Gagan Yaan ‘mission. Since humans will be inside the rocket, we want only non-hazardous fuel and not the hazardous one to power the human space mission's Gagan Yaan rocket," ISRO chief K Sivan, who is also Secretary in department of space, told IANS.

2.1.4 LIFT OFF thanks to Hydrogen peroxide

In 2011 a Soyuz-ST carrier rocket was launched from the European spaceport Kourou in French Guyana for the first time with PROPULSE®, the hydrogen peroxide developed by Evonik for propulsion systems. “And even today our hydrogen peroxide is used in the turbopumps to transport the actual fuels, jet fuel and liquid oxygen, into the combustors under high pressure,” says the 35-year-old chemist. In a sense, then, hydrogen peroxide supplies the energy to feed the huge rocket with fuel. A few metric tons are needed to send the 300-ton Soyuz rocket into orbit.

Despite its high energy density, H2O2 is easy to handle and readily initiates the decomposition processes needed for propulsion technology. Over the last few years the HYPROGEO project funded by the European Union has been explicitly studying hydrogen peroxide as a powerful, safe, and environmentally friendly fuel for space travel. The aqueous solution would then not only start up the turbopump but, along with a second fuel, be responsible for the propulsion of the entire rocket.

“Propulsion is almost always produced by a combustion, and that needs oxygen; this can be supplied by H2O2 in its decomposition process,” says Christ. “The result would be hybrid rockets, relying on hydrogen peroxide and, for example, polyethylene as fuels. The latter is a simple plastic, used for example to make shopping bags.”

2.1.5 Hydrogen Peroxide powered vehicle

When it comes to racing, H2O2 has been used as fuel for rocket cars since Dick Keller built the environmentally clean X-1 in 1965. His Blue Flame car later went 630.388 mph, setting a World Land Speed Record on the Bonneville Salt Flats that lasted from 1970 to 1983. In December 1976, Kitty O’Neil took her H2O2-powered three-wheeled rocket car called the SMI Motivator through Oregon’s Alvord Desert with an average speed of 512.710 mph. O’Neil’s runs reportedly used 60 percent of the available thrust and Kitty was quoted as saying that she could have reached over 700 mph with full power. The following year, Kitty clocked the quickest quarter mile in FIA history of 3.22 seconds at a top speed of 412 mph, in a H2O2 rocket car called the Pollution Packer.

2.1.6 Comparison of hydrogen peroxide with other propellants:

Table 1: R-phrase categories for hydrogen peroxide, hydrazine and (MMH)

| R-Values | H2O2 | Hydrazine | MMH |
| --- | --- | --- | --- |
| R23-28 | 0 | 1 | 1 |
| R39 | 0 | 0 | 0 |
| R48-68 | 0 | 0 | 0 |
| R45-49 | 0 | 1 | 1 |
| R46 | 0 | 0 | 0 |
| R60-61 | 0 | 1 | 1 |
| R50-53 | 0 | 0 | 0 |
| R54-59 | 0 | 0 | 0 |
| Total | 0 | 3 | 3 |

- The findings for R-phrases for hydrogen peroxide, hydrazine, and hydrazine derivatives are summarized in Table 1.

- If a chemical has an R-phrase in one of the categories, it is assigned the number 1.

- If no R-phrases are supplied for that category, it gets a value of zero.

- The total R-values of hydrogen peroxide is “0”

- It typically means that the substance is considered to have minimal or no significant risks under the specific conditions of use or concentration.

CHAPTER 3. PROBLEM STATEMENT

The aerospace industry has long relied on traditional chemical propellants, particularly hydrazine and its derivatives, due to their high energy density, reliability, and ability to perform across a wide range of applications, from spacecraft propulsion to small-scale rocket systems. Despite these advantages, traditional propellants come with a host of significant drawbacks that have raised critical concerns regarding their safety, environmental impact, and economic feasibility.

Hydrazine-based propellants are highly toxic, carcinogenic, and pose serious risks during handling, transportation, and storage. The stringent safety protocols required to mitigate these risks lead to increased operational costs, complex infrastructure requirements, and limited accessibility for smaller-scale aerospace ventures or emerging players in the industry. Furthermore, the combustion of traditional propellants often produces harmful byproducts that contribute to air and ground pollution, compounding the environmental footprint of aerospace activities. As the global push for sustainability intensifies, the continued reliance on these hazardous materials is increasingly being scrutinized.

Moreover, the inherent challenges of traditional propellants extend beyond safety and environmental considerations. These propellants require specialized systems for ignition, precise mixing, and combustion, which can complicate engine design and increase the risk of failures. Additionally, the high volatility of some traditional propellants creates operational hazards, further emphasizing the need for safer alternatives.

To address these multifaceted challenges, the concept of "green propellants" has emerged as a promising avenue for revolutionizing propulsion systems. Among the leading candidates, hydrogen peroxide (H₂O₂) stands out due to its unique properties. Hydrogen peroxide is non-toxic, environmentally benign, and decomposes into harmless byproducts—water and oxygen—making it a far safer and greener alternative to traditional propellants. Its versatility as both a monopropellant and a bipropellant oxidizer open up a range of possibilities for use in modern propulsion systems.

Despite these advantages, the adoption of hydrogen peroxide is not without its own challenges. High-concentration H₂O₂ requires careful handling to prevent decomposition and ensure long-term stability. The efficiency of H₂O₂-based systems heavily depend on the performance of catalysts used to decompose the peroxide into high-temperature gases. Furthermore, achieving performance parity with traditional propellants, especially in terms of specific impulse and thrust, demands significant innovation in engine design and material selection.

CHAPTER 4.  METHODOLOGY

4.1 Performance Optimization Using Simulation Tools

Simulation tools like NASA CEA and Computational Fluid Dynamics (CFD) to model and refine system performance.

- Thermochemical Analysis: Use NASA CEA to calculate optimal operating conditions, including oxidizer-to-fuel ratios, chamber pressures, and exhaust velocities.

4.2 Environmental and Cost Benefits

- Eco-Friendly Manufacturing: Develop scalable and environmentally friendly production methods for high-concentration H₂O₂, reducing its carbon footprint.

- Lifecycle Analysis: Conduct comprehensive assessments to demonstrate the reduction in emissions, health risks, and operational costs compared to traditional propellants.

- Cost Reduction Strategies: Explore mass production and supply chain improvements to lower the cost of H₂O₂ and associated components, making it a more accessible alternative.

CHAPTER 5. RESULT AND DISCUSSION

CEA analysis has been performed using hydrogen peroxide as the fuel

- fuel H2O2(L)

- oxide O2

- p,bar = 65

- pi/p= 68.027

THEORETICAL ROCKET PERFORMANCE ASSUMING EQUILIBRIUM COMPOSITION DURING EXPANSION FROM INFINITE AREA COMBUSTOR

Obtained values for O/F ratio = 3

Table 2 : Various parameters values at Chamber, throat and exit

| Parameters | Chamber | Throat | Exit |
| --- | --- | --- | --- |
| Pinf/P | 1.0000 | 1.8586 | 68.027 |
| P, BAR | 65.000 | 34.973 | 0.95550 |
| T, K | 660.47 | 565.26 | 323.41 |
| RHO, KG/CU M | 3.4345 | 2.1592 | 1.13960 |
| H, KJ/KG | -1380.14 | -1488.80 | -1891.91 |
| U, KJ/KG | -1569.39 | -1650.78 | -1975.75 |
| G, KJ/KG | -5850.40 | -5314.70 | -4080.82 |
| S, KJ/(KG)(K) | 6.7683 | 6.7683 | 6.7683 |
| M, (1/n) | 29.017 | 29.017 | 32.070 |
| MW, MOL WT | 29.017 | 29.017 | 32.070 |
| (dLV/dLT) t | -1.000000 | -1.000000 | -1.15004 |
| (dLV/dLT) p | 1.00000 | 1.00000 | 3.3966 |
| Cp, KJ/(KG)(K) | 1.1574 | 1.1249 | 11.1095 |
| GAMMAs | 1.3290 | 1.3418 | 1.1353 |
| SON VEL, M/SEC | 501.5 | 466.2 | 308.5 |
| MACH NUMBER | 0.000 | 1.000 | 3.279 |

Table:3. PERFORMANCE PARAMETERS

| Parameters | Chamber | Throat | Exit |
| --- | --- | --- | --- |
| Ae/At | - | 1.0000 | 8.7308 |
| CSTAR, M/SEC | - | 645.7 | 645.7 |
| CF | - | 0.7219 | 1.5667 |
| Ivac, M/SEC | - | 813.6 | 1094.6 |
| Isp, M/SEC | - | 466.2 | 1011.7 |

Table:4. Mass fraction

|  | Chamber | Throat | Exit |
| --- | --- | --- | --- |
| H2O2 | 0.13241 | 0.13241 | 0.07329 |
| *O2 | 0.86759 | 0.86759 | 0.86759 |
| H2O (L) | 0.0000 | 0.0000 | 0.05912 |

Table 5: Tabulation for O/F, Isp and Exit Temperature

| O/F | Isp | Temp |
| --- | --- | --- |
| 3 | 1010 | 323 |
| 3.2 | 1000 | 321 |
| 3.4 | 990 | 319 |
| 3.6 | 981 | 317 |
| 3.8 | 972 | 314 |
| 4 | 964 | 312 |
| 4.2 | 956 | 310 |
| 4.4 | 949 | 307 |
| 4.6 | 942 | 305 |
| 4.8 | 936 | 302 |
| 5 | 930 | 299 |
| 5.2 | 925 | 297 |
| 5.4 | 920 | 294 |
| 5.6 | 915 | 291 |
| 5.8 | 910 | 288 |
| 6 | 905 | 284 |
| 6.2 | 901 | 281 |
| 6.4 | 897 | 277 |
| 6.6 | 893 | 274 |
| 6.8 | 889 | 273 |
| 7 | 885 | 273 |
| 7.2 | 882 | 273 |
| 7.4 | 878 | 271 |
| 7.6 | 875 | 268 |
| 7.8 | 872 | 264 |
| 8 | 869 | 260 |

This tabular column is representing the exit O/F ratios, Isp and Exit temperature and Green colored boxes are representing the higher values and the orange-colored boxes are representing the lower values.

Effect of the Oxidizer to Fuel Ratio on the Combustion Performances

Oxidizer to fuel ratio has a significant effect on the combustion chamber temperatures and specific impulse. To begin, combustion chamber temperature has drastically increased for all three combinations of the fuels to the stoichiometric ratio and decreased steadily after the stoichiometric ratio. Further, it is seen that at stoichiometric ratio the chamber temperature and the specific impulse have the highest values among other ratios.

Graph 1:

The graph illustrates the relationship between the oxidizer-to-fuel (O/F) ratio and the specific impulse (Isp) of a rocket engine at sea level. The specific impulse, measured in seconds, represents the efficiency of the rocket propulsion system. The graph shows a decreasing trend, where Isp decreases as the O/F ratio increases. This implies that lower O/F ratios provide higher propellant efficiency at sea level.

Results from the Graph:

At an O/F ratio of approximately 2, the Isp reaches its maximum value of about 1020 seconds. As the O/F ratio increases to around 8, the Isp decreases significantly to roughly 860 seconds. This suggests that an optimal O/F ratio exists (close to 2) for maximizing rocket engine efficiency at sea level, while higher O/F ratios lead to reduced propulsion efficiency.

Graph 2:

The graph represents the relationship between the oxidizer-to-fuel (O/F) ratio and the temperature of the combustion process, measured in Kelvin (K). As the O/F ratio increases, the temperature decreases steadily. This indicates that a higher proportion of oxidizer relative to fuel reduces the combustion temperature, potentially due to incomplete combustion or excess oxidizer absorbing heat.

Results from the Graph:

The temperature remains relatively stable at around 340 K when the O/F ratio is between 2 and 4. However, as the O/F ratio increases beyond 4, the temperature begins to decline more noticeably, reaching approximately 260 K at an O/F ratio of 8. This demonstrates that the temperature is optimal at moderate O/F ratios and decreases as the ratio becomes more oxidizer-heavy.

CHAPTER 6. CONCLUSION

From the graphs, it can be concluded that the efficiency of a rocket engine at sea level is highly dependent on the oxidizer-to-fuel (O/F) ratio. The specific impulse (Isp) is maximized at an O/F ratio near 2, indicating that this is the optimal ratio for achieving the best propellant performance. As the O/F ratio increases beyond this point, the specific impulse decreases, showing diminishing returns in propulsion efficiency when the oxidizer content is higher than optimal.

And it is evident that the combustion temperature also decreases as the O/F ratio increases. This suggests that excess oxidizer in the mixture may dilute the heat generated during combustion, reducing the thermal energy available. An optimal O/F ratio exists where the temperature is relatively higher, supporting efficient combustion and higher Isp. These findings highlight the need for careful optimization of the O/F ratio to balance maximum efficiency and combustion temperature for effective rocket engine operation.

Analysis was performed taking into account all compositions and comparing combustion products to obtain the maximum performance at the correct O/F ratio and fixed chamber pressure. It is observed that hydrogen peroxide has remarkable advantages similar to other propellants and it has fewer metallic oxides at the exit, which makes hydrogen peroxide the rewarding green propellant for the next generation.

REFERENCES

- Tamma, Paola; Schaart, Eline; Gurzu, Anca (11 December 2019). "Europe's Green Deal plan unveiled". POLITICO. Retrieved 29 December 2019.

- ^ Jump up to:a b c Simon, Frédéric (11 December 2019). "EU Commission unveils 'European Green Deal': The key points". euractiv.com. Retrieved 29 December 2019.

- "Carbon Border Adjustment Mechanism". European Parliament.

- ^ Jump up to:a b c (COM/2020/98 final) COMMUNICATION FROM THE COMMISSION TO THE EUROPEAN PARLIAMENT, THE COUNCIL, THE EUROPEAN ECONOMIC AND SOCIAL COMMITTEE AND THE COMMITTEE OF THE REGIONS — A new Circular Economy Action Plan For a cleaner and more competitive Europe

- ^ "Circular economy action plan". ec.europa.eu. Retrieved 30 May 2022.

- [4] GRASP team. General assessment of green propellants, seventh framework program, 2008.

- [5] Green Hydrogen Peroxide (H2O2) monopropellant with advanced catalytic beds. www.esa.int/About_Us/Business_with_ESA/Small_and_Medium_ Sized_Enterprises/SME_Achievements/Green_Hydrogen_Peroxide_H2O2_ monopropellant_with_advanced_catalytic_beds. Accessed: 2019-12-27.

- [6] Green propulsion system. https://www.nammo.com/news-and-events/news/ green-propulsion-system/. Accessed: 2020-01-07.

- [7] ”Nammo signs its largest ever space development contract” . https://www.epicos.com/article/413522/ nammo-signs-its-largest-ever-space-development-contract. Accessed: 2020-01-07.

- [8] HYPROGEO project completion . https://ilot.edu.pl/en/ hyprogeo-project-completion/. Accessed: 2020-01-07.